LAB EXPERIMENTS

NAME: NISHEVITHA SRI.S

REG NO: 192511424

SLOT: D

COURSE CODE: ECA1415

COURSE: EMBEDDED SYSTEM

EXPERIMENT:11

STUDY OF GPIO &AMP; BIT ADDRESSING IN AT89C51

PROGRAM:

#include <reg51.h>

sbit LED1 = P1^0;

sbit LED2 = P1^1;

void delay ()

{

unsigned int i, j;

for (i = 0; i < 200; i++)

{

for (j = 0; j < 1000; j++);

}

}

void main ()

{

while (1)

{

LED1 = 1;

LED2 = 0;

delay ();

LED1 = 0;

LED2 = 1;

delay ();

}

}

OUTPUT:

EXPERIMENT: 12

EMBEDDED C PROGRAM TO MULTIPLY TWO 16-BIT BINARY

NUMBERS

PROGRAM:

#include <reg51.h>

void main ()

{

unsigned int num1, num2;

unsigned long int product;

while (1)

{

num1 = 0x2222;

num2 = 0xBBBB;

product = (unsigned long int) num1 * num2;

P0 = product & 0xFF;

P1 = (product & 0xFF00) >> 8;

P2 = (product & 0xFF0000) >> 16;

P3 = (product & 0xFF000000) >> 24;

}

}

OUTPUT:

EXPERIMENT: 13

EMBEDDED C PROGRAM TO ADD AN ARRAY OF 16-BIT NUMBERS

AND STORE THE 32-BIT RESULT IN INTERNAL RAM

PROGRAM:

#include <reg51.h>

void main(void)

{

unsigned int i;

unsigned int array [5] = {0x1111, 0x2222, 0x8888, 0x4444, 0xABCD};

unsigned long int sum = 0;

for (i = 0; i < 5; i++)

{

sum = sum + array[i];

}

P0 = (unsigned char) (sum & 0xFF);          // Lower 8 bits

P1 = (unsigned char) ((sum >> 8) & 0xFF);   // Next 8 bits

P2 = (unsigned char) ((sum >> 16) & 0xFF);  // Upper 8 bits

While (1);

}

OUTPUT:

EXPERIMENT: 14

EMBEDDED C PROGRAM TO DISPLAY “HELLO WORLD”

MESSAGE

PROGRAM:

#include <reg51.h>

#include <stdio.h>

void main(void)

{

SCON = 0x50;    // UART mode 1, 8-bit data, enable receiver

TMOD = 0x20;    // Timer 1, Mode 2 (8-bit auto reload)

TH1 = 0xFD;     // Reload value for 9600 baud rate

TR1 = 1;        // Start Timer 1

TI = 1;         // Set TI to send the first character

while (1)

{

printf ("Hello World!\n");

}

}

OUTPUT:

EXPERIMENT: 15

EMBEDDED C PROGRAM TO CONVERT THE HEXADECIMAL DATA 0XCFH TO DECIMAL AND DISPLAY THE DIGITS ON PORTS P0, P1 AND P2

PROGRAM:

#include <reg51.h>

void main(void)

{

unsigned char hexa = 0x08;

unsigned char hundreds, tens, units;

ACC = hexa;

B = 10;

ACC = ACC / B;

units = B;

B = 10;

ACC = ACC / B;

tens = B;

hundreds = ACC;

P0 = units;

P1 = tens;

P2 = hundreds;

while (1);

}

OUTPUT:

EXPERIMENT: 16

SERIAL INTERRUPT USING UART (KEYBOARD INPUT) IN

KEIL

PROGRAM:

#include <reg51.h>

#include <stdio.h>

void serial_ISR(void) interrupt 4

{

char ch;

if(RI)

{

RI = 0;              // Clear receive interrupt flag

ch = SBUF;           // Read received character

printf("\n>>> INTERRUPT RECEIVED: ");

printf("%c\n", ch);

}

}

void delay(void)

{

unsigned int i;

for (i = 0; i < 50000; i++);   // Delay for visibility

}

void main(void)

{

SCON = 0x50;      // UART mode 1, 8-bit, REN enabled

TMOD = 0x20;      // Timer1 Mode 2 (Auto Reload)

TH1 = 0xFD;       // Baud rate 9600

TR1 = 1;          // Start Timer1

IE = 0x90;        // EA = 1, ES = 1 (Enable Serial Interrupt)

TI = 1;           // Enable printf

while (1)

{

printf ("Hello World\n");

delay ();

}

}

OUTPUT:

EXPERIMENT: 17

UART INTERFACING USING INTERRUPT IN

KEIL

PROGRAM:

#include <reg51.h>

void serial_ISR(void) interrupt 4

{

if(RI)

{

P1 = SBUF;    // Display received data on Port 1

RI = 0;       // Clear receive interrupt flag

}

}

void main(void)

{

TMOD = 0x20;      // Timer1 Mode 2 (8-bit auto reload)

TH1  = 0xFD;      // 9600 baud rate

SCON = 0x50;      // UART Mode 1, 8-bit, enable receiver

TR1  = 1;         // Start Timer1

ES = 1;           // Enable Serial Interrupt

EA = 1;           // Enable Global Interrupt

while(1);

}

OUTPUT:

EXPERIMENT: 18

GENERATION OF PWM USING TIMER

INTERRUPT

PROGRAM:

#include <reg51.h>

sbit PWM = P1^0;

unsigned char count = 0;

unsigned char duty = 100;    // Change this value (0–200)

void timer0_ISR(void) interrupt 1

{

TH0 = 0xFC;               // Reload for 1 ms delay

TL0 = 0x66;

count++;

if(count < duty)

PWM = 1;              // ON time

else

PWM = 0;              // OFF time

if(count >= 200)          // Total period = 200 ms

count = 0;

}

void main(void)

{

TMOD = 0x01;              // Timer0 Mode 1 (16-bit)

TH0 = 0xFC;               // 1 ms preload

TL0 = 0x66;

IE = 0x82;                // EA = 1, ET0 = 1

TR0 = 1;                  // Start Timer0

while(1);

}

OUTPUT: